Assignment #1

# Issued: 2 April 2020 Due: 20 April 2020 (at 23h59, eastern standard time)

Please read each question carefully, be careful with units, and answer *all* parts. All figures must be clearly labeled: axes include variables being plotted and their units as necessary. Show your work, including the equations used, scripts used for numerical calculations, and any Smith charts you may have used.

Antoine Wang 260766084

1. In this question, you will study reflection at normal incidence from multi-layered media. Consider two media:

- = air, with the material properties σ = 0, ε = ε0 , μ = μ0

- = plastic, with the material properties σ = 0, ε = 3ε0 , μ = μ0

For simplicity, assume that all material parameters are independent of frequency.

- What is the reflection coefficient Γ for a normally incident wave from material A to material B? What is the reflection coefficient Γ for a normally incident wave from material B to material A? [2pts]

From A to B:

Γ = ηB - ηA ηB+ ηA=3773 - 377 3773 + 377= -0.267949

From B to A:

Γ = ηA - ηB ηA+ ηB=-3773+ 377 3773 + 377= 0.267949

- Calculate the reflection coefficient Γ for an ABA structure as shown below, with the layer thickness d1 = 0.010 m, for the frequency range 0 < f < 10 GHz with frequency steps of Δf = 0.05 GHz. Plot the resulting reflection amplitude |Γ| versus frequency f. [2pts]

Procedure:

Coding of the solution in MATLAB:

clear; close all;

freq =0:0.05e9:10e9;

nb = 377/sqrt(3);

na = 377;

reCoefficient1 = 0.267949;

beta = 5.777e-9;

impedence = zeros(length(freq));

reCoefficient2 = zeros(length(freq));

for k=1: length(freq)

impedence(k) = nb .* (1+reCoefficient1.*exp(-2j.*beta.*(2*pi).*freq(k).*0.01))./(1-reCoefficient1.*exp(-2j.*beta.*(2*pi).*freq(k).*0.01));

reCoefficient2(k) = abs((impedence(k) - na)/(impedence(k) + na));

end

plot(freq,reCoefficient2)

title('Plot of reflection amplitude, question 1.b')

Plotting:

- Consider your answer to part b). What is the lowest frequency f where the thickness d1 is equal to λ/2 in layer B (ie. layer B acts as a ½ wave plate) ? What is the lowest frequency f where the thickness d1 is equal to λ/4 in layer B (ie. layer B acts as a ¼ wave plate) ? [2pts]

According to the plotting in B, the lowest frequency of ½ wave frequency is 8.7Ghz. The reason is that the reflection coefficient reaches zero due to a ABA structure.

The lowest frequency for the thickness of b to be ¼ wavelength is thus 4.35Ghz, half of that of ½ wavelength frequency.

- Calculate the reflection coefficient Γ for an ABABA structure as shown below, with the layer thickness d1 = 0.010 m and d2 = 0.017 m for the frequency range 0 < f < 10 GHz with frequency steps of Δf = 0.05 GHz. Plot the resulting reflection amplitude |Γ| versus frequency f. [2pts]

Solution:

This structure is simply adding another AB layer to the left of the original ABA structure. Thus in MATLAB code simulating the procedure, adding two extra steps in d2 of A and d1 of B simulates the condition of this question.

MATLAB code:

freq =0:0.02e9:10e9;

nb = 377/sqrt(3);

na = 377;

reCoefficient1 = 0.267949;

betaB = 5.777e-9;

betaA = 3.333e-9;

impedenceB1 = zeros(length(freq));

impedenceB2 = zeros(length(freq));

impedenceA = zeros(length(freq));

reCoefficient2 = zeros(length(freq));

reCoefficient3 = zeros(length(freq));

reCoefficient4 = zeros(length(freq));

for k=1: length(freq)

% Original ABA---------------

impedenceB1(k) = nb .* (1+reCoefficient1.*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01))./(1-reCoefficient1.*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01));

reCoefficient2(k) = (impedenceB1(k) - na)/(impedenceB1(k) + na);

% Adding additional A of 0.17m--------

impedenceA(k) = na .* (1+reCoefficient2(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017))./(1-reCoefficient2(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017));

reCoefficient3(k) = (impedenceA(k) - nb)/(impedenceA(k) + nb);

% repeat with a B layer of 0.1m

impedenceB2(k) = nb .* (1+reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01))./(1-reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01));

reCoefficient4(k) = abs((impedenceB2(k) - na)/(impedenceB2(k) + na));

end

plot(freq,reCoefficient4)

title('Plot of reflection amplitude, question 1.d');

Plot:

- Calculate the reflection coefficient Γ for an ABABABA structure as shown below, with the layer thickness d1 = 0.010 m and d2 = 0.017 m, for the frequency range 0 < f < 10 GHz with frequency steps of Δf = 0.05 GHz. Plot the resulting reflection amplitude |Γ| versus frequency f. [2pts]

Solution:

Repeat the procedure of 1.d by adding another AB layer to the ABABA structure.

Matlab code:

freq =0:0.02e9:10e9;

nb = 377/sqrt(3);

na = 377;

reCoefficient1 = 0.267949;

betaB = 5.777e-9;

betaA = 3.333e-9;

impedenceB1 = zeros(length(freq));

impedenceB2 = zeros(length(freq));

impedenceA = zeros(length(freq));

reCoefficient2 = zeros(length(freq));

reCoefficient3 = zeros(length(freq));

reCoefficient4 = zeros(length(freq));

for k=1: length(freq)

impedenceB1(k) = nb .* (1+reCoefficient1.*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01))./(1-reCoefficient1.*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01));

reCoefficient2(k) = (impedenceB1(k) - na)/(impedenceB1(k) + na);

impedenceA(k) = na .* (1+reCoefficient2(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017))./(1-reCoefficient2(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017));

reCoefficient3(k) = (impedenceA(k) - nb)/(impedenceA(k) + nb);

impedenceB2(k) = nb .* (1+reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01))./(1-reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01));

reCoefficient4(k) = (impedenceB2(k) - na)/(impedenceB2(k) + na);

% from this point we are adding a additional AB layer on to ABABA

% structure in Question d

impedenceA(k) = na .* (1+reCoefficient4(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017))./(1-reCoefficient4(k).*exp(-2j.*betaA.*(2*pi).*freq(k).*0.017));

reCoefficient3(k) = (impedenceA(k) - nb)/(impedenceA(k) + nb);

impedenceB2(k) = nb .* (1+reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01))./(1-reCoefficient3(k).*exp(-2j.*betaB.*(2*pi).*freq(k).*0.01));

reCoefficient4(k) = abs((impedenceB2(k) - na)/(impedenceB2(k) + na));

end

plot(freq,reCoefficient4)

title('Plot of reflection amplitude, question 1.e')

Plot:

Optional exercise for self-education beyond ECSE 354 (no marks): Derive an analytical formula for the reflection coefficient Γ for an arbitrarily long stack A(BA)n = A BA BA BA …. BA structure for n ≥ 1.

2. In this question, you will consider the properties of a WR28 hollow metallic waveguide. The waveguide dimensions are a = 7.11 mm and b = 3.56 mm. The inner walls of the waveguide are gold plated, with a conductivity σ = 4.1 x 107 S/m and a permeability μ = μ0. The waveguide is filled with air, which can be approximated as vacuum. Recall the vacuum speed of light is c = (ε0μ0)-1/2.

- What is the cut-off frequency fc for the TE10 mode? [1pt]

- Calculate the angle of incidence θ for the TE10 mode, as defined in the plane wave picture of TEm0 modes, for the frequency range fc < f < 2fc . Plot θ versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

cos_incident = zeros(length(freq));

incident = zeros(length(freq));

for k=1: length(freq)

cos_incident(k) = (pi/7.11e-3)/(2*pi*freq(k)*3.33e-9);

incident(k) = acosd(cos_incident(k));

end

plot(freq,incident)

Plot:

- Calculate the phase constant β = kz for the TE10 mode, as defined in the plane wave picture, for the frequency range fc < f < 2fc . Plot cβ/ω versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

beta = zeros(length(freq));

cb_w = zeros(length(freq));

for k=1: length(freq)

beta(k) = 2*pi*freq(k)/3e8*sqrt(1 - (fc^2 / freq(k)^2));

cb_w(k) = beta(k)*3e8/2/pi/freq(k);

end

plot(freq,cb_w)

title('Plot of question 2.c')

Plot:

- Calculate the phase velocity vp for the TE10 mode for the frequency range fc < f < 2fc . Plot vp/c versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

betacb_w = zeros(length(freq));

vp_c = zeros(length(freq));

for k=1: length(freq)

betacb_w(k) = sqrt(1 - (fc^2 / freq(k)^2));

vp_c(k) = 1/betacb_w(k);

end

hold on

plot(freq,vp_c)

title('Plot of phase velocity, question 2.d')

Plot:

- Calculate the group velocity vg for the TE10 mode for the frequency range fc < f < 2fc . Plot vg/c versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

% vg = dw/db

% the formula is c*sqrt(1 - (fc^2 / freq(k)^2))

vg_c = zeros(length(freq));

for k=1: length(freq)

vg_c(k) = sqrt(1 - (fc^2 / freq(k)^2));

end

hold on

plot(freq,vg_c)

title('Plot of group velocity, question 2.e')

Plot:

- Calculate the sheet resistance Rs for gold for the frequency range fc < f < 2fc . Plot Rs versus f.

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

mu = 4e-7*pi;

sigma = 4.1e7;

Rs = zeros(length(freq));

for k=1: length(freq)

Rs(k) = sqrt(pi*mu*freq(k)/sigma);

end

plot(freq,Rs)

title('Plot of Rs, question 2.f')

Plot:

- Calculate the attenuation constant α for the TE10 mode for the frequency range fc < f < 2fc . Plot α versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

mu = 4e-7*pi;

sigma = 4.1e7;

Rs = zeros(length(freq));

alpha = zeros(length(freq));

vg = zeros(length(freq));

for k=1: length(freq)

Rs(k) = sqrt(pi*mu*freq(k)/sigma);

vg(k) = 3e8*sqrt(1 - (fc^2 / freq(k)^2));

alpha(k) = Rs(k)*(1/3.56e-3+2*(fc^2 / freq(k)^2)/7.11e-3)/ mu/ vg(k);

end

plot(freq,alpha)

title('Plot of alpha, question 2.g')

Plot:

- Calculate the loss tangent tan(δD) for the frequency range fc < f < 2fc . Plot tan(δD) versus f. [2pts]

Matlab code:

%calculating the cutoff frequency

fc = 1*3e8/2/7.11e-3;

freq =fc:0.05e9:2*fc;

beta = zeros(length(freq));

Rs = zeros(length(freq));

alpha = zeros(length(freq));

vg = zeros(length(freq));

tan_delta = zeros(length(freq));

mu = 4e-7*pi;

sigma = 4.1e7;

for k=1: length(freq)

beta(k) = 2*pi*freq(k)/3e8*sqrt(1 - (fc^2 / freq(k)^2));

Rs(k) = sqrt(pi*mu*freq(k)/sigma);

vg(k) = 3e8*sqrt(1 - (fc^2 / freq(k)^2));

alpha(k) = Rs(k)*(1/3.56e-3+2*(fc^2 / freq(k)^2)/7.11e-3)   / mu / vg(k);

% Good dielectric approximation links alpha, beta and loss tangent

% together

tan_delta(k) = 2*alpha(k)/beta(k);

end

plot(freq,tan_delta)

title('Plot of question 2.h')

Plot:

Optional exercise for self-education beyond ECSE 354 (no marks): Consider a closed rectangular metal box with dimension a in the x direction, b in the y direction, and c in the z direction. Assume c = a, a > b, the walls are perfect electric conductors, and the interior is filled with air. This is called a resonant cavity. Derive an analytical formula for the lowest frequency solution E(x,y,z) and the corresponding frequency fc of this solution.

3. In this question, you will explore the vector potential A with the Lorenz gauge condition.

- Consider a plane wave propagating in a lossless medium. Give an expression for the time average Poynting vector in terms of the vector potential phasor A, wavevector k, and physical constants as necessary. [1½ pts]

Poynting vector Is in the same direction as the wave propagates, thus

Sr=-ω22ηA02 n

- A plane wave is propagating in vacuum in the +z direction at a frequency f = 20 GHz with electric field linearly polarized in the x-direction with 1 V/m amplitude. Give the phasor expressions for the vector potential A, electric field E, and magnetic field H. [1½ pts]

- A plane wave is propagating in vacuum in the +z direction at a frequency f = 20 GHz with electric field circularly polarized ( polarization vector 𝐚  ) with 1 V/m amplitude. Give the phasor

expressions for the vector potential A, electric field E, and magnetic field H. [1½ pts]

- Consider two plane waves at a frequency f = 20 GHz propagating in vacuum, one in the +z direction with electric field polarization vector 𝐚  and another in the -z direction with electric field polarization vector 𝐚 , and each with a 1 V/m electric field amplitude. Give the phasor expressions for the total vector potential A, total electric field E, and total magnetic field H. [1½ pts]

Optional exercise for self-education beyond ECSE 354 (no marks): Ampere’s law can be easily used to derive the magnetic flux density B around a wire carrying a dc current I0. Find a vector potential A that leads to the magnetic flux density 𝐁 = ∇ × 𝐀 in this problem. Can you derive this vector potential from a current integral? (warning: this problem is deceptively simple, but rather advanced)

4. Consider a dipole antenna of length Δℓ = 10 cm operating at a frequency f = 500 MHz. Approximate the dipole antenna behavior as that of a Hertzian dipole. Air is the surrounding medium and conductor resistance can be neglected.

- What is the ratio of antenna length to vacuum wavelength, Δℓ/λ0, for this antenna? [1pt]

- What is the radiation resistance Rrad ? [1pt]

- If the voltage phasor exciting the antenna is V = 1 V, how much power does the antenna radiate? [1pt]

Solution for a-c:

- Assume the antenna is oriented along the z axis, located at the origin r0 = (0,0,0), and excited with a voltage phasor V = 1 V. What is the vector potential phasor A, electric field phasor E, magnetic field phasor H, and time average Poynting vector amplitude S at the position 𝐫𝟏 = 100 m 𝐱, at the position 𝐫𝟐 = 100 m 𝐲 and at the position  𝐫𝟑 = 100 m 𝐳 . Consider carefully whether a far-field approximation can be used. Give your answers using Cartesian unit vectors 𝐱, 𝐲 and 𝐳. [4pts]

Solution for r1 = 100mx:

Solution for r2 = 100my:

Solution for r3 = 100mz:

- Including reactance, the antenna impedance is ZL = Rrad – j 220 Ω. Design a shorted stub transmission line circuit to achieve an impedance match between a lossless transmission line and the antenna. The transmission line characteristic impedance is Z0 = 100 Ω and the transmission line phase velocity is vp = 2x108 m/s. Provide the distance d between the antenna load and the junction stub, and the length of the shorted stub l. If you find multiple solutions, identify the most physically compact solution. [3pts]

Optional exercise for self-education beyond ECSE 354 (no marks): Consider a Hertzian dipole oriented located at a position  𝐫𝟎 = 𝑧 𝐳  above a perfect electric conductor at the z = 0 plane. Identify the minimum distance and orientation of the dipole that would maximize emission in the vertical (+z) direction. This problem is important for understanding antenna interaction with conductive objects. HINT: consider using the concept of image charge from electrostatics.